# Comparison analysis of chattering in smooth sliding mode controlled dc-dc buck converter using constant plus proportional  reaching law and proposed reaching laws

Siddesh K.B1, Dr.Basavaraja Banakara2, R.Shivarudraswamy3

1, Department of Electronics and Communication Engineering, SJMIT, VT University, Belagavi

2Department of Electrical & Electronics Engineering, UBDTCE, Davangere, VTUniversity

2Department of Electrical & Elctronics Engineering, MIT, Manipal, Karnataka

| Article Info |  | ABSTRACT (10 PT) |
| --- | --- | --- |
| Article history: ReceivedJun 9, 2016 Revised Nov 20, 2016 Accepted Dec 11, 2016 |  | This paper presents a comparison analysis of chattering in sliding mode controlled dc-dc buck converter for chattering suppression using constnt plus proportionl reaching law and proposed reaching laws. A smooth SMC is used for chattering suppression in dc-dc buck converter. The different swicthing functions are used in proposed reaching laws and constant plus proportional reaching law applied to SMC DC-DC buck converter, the tan hyperbolic reaching law, sigmoid reaching law and constant plus propostional rate reaching law. The proposed methods tan hyperbolic gives less switching losses among the reaching laws and stable output voltage. Inturn performance of the dc-dc buck converter increases, tanhyperbolice reaching law is more sensitive to matched and mismatched disturbance and parameter uncetainities.Loading conditions are also applied to dc-dc buck converter to mesuare the disturabnces and parametric variations. The results are verified by MATLAB/Simulink. |
| Keywords: Sliding mdoe control Tan hyperbolic reaching law Sigmoid reaching law Chattering Reaching law |  |  |
|  |  | Copyright © 2019Institute of Advanced Engineering and Science. All rights reserved. |
| Corresponding Author: K.B.Siddesh, Department of Elctronics and communication Engg. SJMIT, VTU Belagavi.577501 Email: kondapur.b@gmail.com |  |  |

- INTRODUCTION

DC-DC converters are operated at very high switching frequencies, and this allows the use of small inductive and capacitive components which increases the dynamic behavior and alleviate the size of the converter. The dc-dc convertetr working with a sliding mode controller based reaching law. Even though many parameters influences the operating points of the converters.1Line and Load variation 2. Electromagnetic interferences (EMI).3Chattering .The SMC is derived from VSCS and applied to many application ,usning reaching laws,switching surfaces, The objective structure of system is changed intentionally during the time with respect to the present structure control law.The traditional reaching laws are explained[1].The applications and survey given in [2],the variable sytructure applied with reaching laws ,it can minimize the chattering in the systems but not removed from the system.The design and analysis of dc-dc converter done using adaptive smc,here not discussed the chattering of converter[3].Using fixed frequency modulation for qusi sliding mode control (QSMC) has been done through experimental and simulation verifications[4].Analysis of new improved reaching law with chattering reduction in smc based to  many applications[5]. In this work  comparative study of sliding mode control (SMC) with PID controller, derivative sliding surface with three different reaching laws, namely, constant  rate, exponential and power rate reaching law to control a two-link robotic manipulator [6]. An improved traditional smc in positive invariant ones, ths sliding surfaces enters by the variables .it will not move away from the sliding line [7].  In this work presents a new handle methods based on a novel reaching law to overcome slow convergence rate and reduce the chattering in sliding mode control of permanent magnet synchronous motors (PMSM). The constant reaching law converges slowly. Although the power reaching law converges fast during large error [8]. This work proposes a novel continuous reaching law for chattering-free sliding mode control by using two hyperbolic functions with like changing rate and contradictory amplitude characteristics. The earliest function is an inverse hyperbolic sine function, which can guarantee the quick convergence as the first value of the sliding mode variable is far away from the equilibrium. When the sliding mode variable is approaching to zero, the second hyperbolic tangent function can ensure the sliding mode variable be infinitely close to zero slightly than cross the zero[9].In this work sigmoid variable rate reaching law applied to the systems and it analysis with conventional  reachin law ,sigmoid reaching law is dynamic and superior than traditional reaching law[10].In this work comaprision between sigmoid and exponential reaching law applied dc-dc buck coveretr,chattering  was minimized[11].This work gives chattering minimization with smc based reaching laws[12].This work presents chattering alleviation using modified power rate reaching law for robotics,.here varying the exponential term of power rate reaching law inorder to minimise the chattering[13].This work presents saturation function  instead of signum function ,so that it covers the entire sliding mode portion and chattering reduces[14].This work presents a improved reaching law in smc for chattering miminsation ,as compare to previous works[15].This reaching law is an adaptive in nature but it implemented to second order nonlinear function[16].This work presents new power exponential rate reaching law for second order system to reduce the chattering in the system and control gains automatically adjusts the switching function[17].In this work discrete sliding mode control and classical smc with relalative degree1 and relative degree2 for dc-dc buck converter discussed ,no chattering issues in this work [18].In this work reaching law based smc for dc-dc buck converter has been done.here d+ and d- method implemented,chattering is alleviated,but not measured and traditional reaching law implemented[19].In this work newly exponential term added to the power reaching law.It  implemented to PMSM sytem to achive better performance, fast convergence  and reduces the chattering in thePMSM[20].

All the above [14, 15, 17, 16] are new reaching law in their differential equations, these are reduces the chattering to certain extent and adapts the smooth variation of switching function.All these new reaching laws reduces the amplitude of the chattering in the system. The problems in the previous work [18,11,19]

Sliding mode portion not covered, Chattering existence,More switching losses,More time to reach the steady state ,Ripple in the output voltage,Less response to external distrubances and uncerainity of the systems.

As many reasearchesrs are discused about the new modified reaching laws, chattering mitigation and their consequences on the systems. As in this work considered a dc-dc buck converter, this converter requires many application such as renewable energy sources, laptops,electronics industries,electronic gadgets etc., using sliding mode controller achieving a  constant output voltage. But more switching losses occur due to chattering in system, inturn high speed switching frequency operation in the swictching devices. This is the problem of the sliding mode controller.In order to above mentioned problem to alleviate the chattering, a proposed  tan hyperbolic reaching law,sigmoid reaching law techniques and constant plus proportional rate reaching law  are applied to buck dc-dc converter.Differnt loading conditions are applied .

2. SMOOTH SLIDING MODE CONTROL FOR DC-DC BUCK CONVERTER USING REACHING

LAW

The every state variable may be affected by an input voltage, if the system is controllable. The converter output voltage and its derivative are both selected as state variables for DC-DC Buck converter. Figure 1 shows the general structure of SMC for DC-DC converters. [5].

Figure1. a Circuit diagram of SMC for DC-DC buck converter.

- The mathematical equation

[5]

(1)

- X1 is the error, X2 is the derivative of the error and X3 is the integral error.The differential of the state variables

- The sliding surface is given by

- (2)

- The derivative of the sliding surface is given by

- [5]           							           (3)

- where α1 α2 &  α3 represent the sliding coefficients, and X1, X2, and X3 denote the state feedback variables to be controlled.

A. SMOOTH SLIDING MODE CONTROL FOR DC-DC BUCK CONVERTERS WITH CONSTANT PLUS PROPORTIONAL RATE REACHING LAW (TRADITIONAL REACHING LAW)

The reaching law is a differential equation which indicates the dynamics of a switching function S(X). It is given by[1]

(4)

Where Q and k are constants Constant plus proportional rate reaching law with the two constants, Q and K helps to obtain fast convergence, but makes the system  increased chattering, but on the other hand it leads disturbances on the sliding mode portion. By adding the proportional rate term –ks, the state is forced to approach the switching surface. [1].

(5)

(6)                               	 			                                         (7)

B. SMOOTH SLIDING MODE CONTROL FOR DC-DC BUCK CONVERTER WITH PROPOSED METHOD 1:  SIGMOID REACHING LAW

Yun-long Liu, et.al in 2010 proposed a Variable Structure Control for Affine Systems. This reaching law proposed in this paper is based on the choice of sigmoid variable term that adapts to the variation of switching function in this reaching law [10]. From [11] applied to the dc-dc bukc converter.here, it has compared with conventional reaching law.Chattering is not measured.In this reaching law sigmoid function can reach the region of the switching surface in finite speed and the stability of origin can be guaranteed,with absolute value of the state variables,  and minimize chattering.

(8)

(9)

Simulation modeling equations

(10)

Where U=Ueq

(11)

Where    Ueq=d;   d=Vc/Vramp;   Vramp=β Vin

(12)

C. SMOOTH SLIDING MODE CONTROL FOR DC-DC BUCK CONVERTER WITH PROPOSED METHOD 2: TAN HYPERBOLIC REACHING LAW

Liang Tao  et al. in 2018. Proposed a novel continuous reaching law for chattering-free sliding mode control by using two hyperbolic functions.[9]

A proposed method 2 reaching law is given by

(13)

m1>0, m2>0,α >0, λ>0

The hyperbolic function is mathematical expressions its range from (-1, +1),

Here tan hyperbolic used instead of sign and saturation function, it adapts switching function as compared to saturation function .The reaching phase and convergence speed of the   hyperbolic function is  fast and keep the  system state at sliding line. . Αs α varies, the system bringing on sliding line and hits the sliding surface at greater velocity.  term keeps system the stable on the phase plane and selecting control gain value  λ such that chattering will be  mitigated at certain point.

Simulation modeling equations

(14)

(15)

4. RESULTS AND DISCUSSION

Table 1 Specification of Sliding Mode Control DC-DC Buck Converter with reaching laws

| Sl.No. | Parameter | Symbol | Value |
| --- | --- | --- | --- |
| 1 | Input voltage | Vi | 24Volts |
| 2 | Capacitance | C | 220μF |
| 3 | Inductance | L | 69μH |
| 4 | Switching Frequency | fs | 200KHz |
| 6 | load resistance | RL(max) | 10 Ohm |
| 7 | Desired Output voltage | Vod | 12V |
| 8 | Reference voltage | Vref | 12V |
| 9 | Inductance | LL | 1mH |
| 10 | Capacitance | CL | 1microF |
| 11 | Feedback factor | β | 0.98 |
| 12 | Sliding coefficients, | α1, α2, α3 | 3,25,2000 |
| 13 | Duty cycle | α | 0.5 |
| 14 | Efficiency of the Converter | η | 0.91 |
| 15 | .λ α |  | 0.5,1 |

Table 1 gives the specifications of the of Sliding Mode Control DC-DC Buck Converter with reaching laws

Figure 2. Shows the output voltage v/s Time in secs of proposed method 1, proposed method 2 and consant plus proportional rate reaching law-(R-Load)

Figure 2. shows that proposed method 1 gives the ripple in the output voltage is 2mVs ,whereas the proposed method 2 tanhyperbolic reaching law gives no ripple in the output voltage.This is due to chattering exists in the sigmoid reaching law.Constnt plus proportional rate reaching law not yet reaache dsteady state becase more chatterin exist in the sliding manifold

Figure 3. Proposed method 1, proposed method 2 and consant plus proportional rate reasciong law- (R-L Load)

Figure 3. shows that the proposed method 2 has s oveshoot voltage of 1.5Vand ripple in the output voltage (2mV), wheras the proposed method 1 gives no overshoot and no ripple in the output voltage and consatnt proportional rate reaching law taken more time to reach the steady state,the more chattering occurs in the Proposed method 2.It is observed that proposed method 2 is a more sensitive for load component variations.

Figure 4.Shows output voltage v/s Time in secs of proposed method 1, proposed method 2 and consant plus proportional rate reasciong law - (R-C Load)

Figure 4 Shows that the proposed method 1 has 0.38V, deviated from the desird output voltage, whereas the proposed method 2 has 0.28V, deviated from the desired output voltage.Constant plus proportional rate reachig law reaches the steady state at 6msecs, but not stable in output voltage.Proposed method 1 has a more sensitive to change in load components and exhibits the change in output voltage.

Figure 5. Shows output voltage v/s Time in secs of proposed method 1, proposed method 2 and consant plus proportional rate reasciong law -(R-L-C Load)

Figure 5 shows that proposed method 1 has deviated the output voltage 0.43V, whereas the proposed method 2 has 0.29V . Constant plus proportional rate reachig law reaches the steady state at 6 msecs, but not stable in output voltage. In RLC load, the proposed method 2 has less deviated from the desired output voltage and more stable.Proposed method 1 has more deviated.

Figure6. Shows derivative of error v/s error of proposed method 1, proposed method 2 and consatnt plus propotional rate reaching law (chattering)

Figure 6 shows that derivative of error and error.The chattering is occurred on nearby the origin of both methods. Whereas the constsnt treaching law is far away from the origin, due to chattering in the sliding manifold.Proposed method1 is just away from the origin, when compare to proposed method 2 and constant plus proportional reaching law.This is due to sign switching function gives discontinuous in the sliding mode, proposed method 2 gives a smoother switching function and avoid the discontinuities in the sliding mode, whereas the proposed method 1 gives less discontinuity in the sliding mode. Hence proposed method 2 exhibits superior continuous approximation than other methods.

| Parameters | A proposed method1 sigmoid reaching law | A proposed method2 tanhyperbolic reaching law | Constant plus proportional rate reaching law |
| --- | --- | --- | --- |
| Chattering amplitude from x and y axis | 0.0165 | 0.0143 | 0.5 |
| Reaching time To steady state | 0.271mSecs | 0.270mSecs | 8mSecs |
| Output Voltage(R load) | 12.065V | 12.01V | 11.97V |
| Output Voltage(R-L load) | 12.025V | 12.012V | 11.97V |
| Output Voltage(R-C load) | 12.32V | 12.36V | 12.01V |
| Output Voltage(R-L-C load) | 12.44V | 12.29V | 11.99V |
| Switching loss | 0.31W | 0.29W | 0.55W |

Table 2: Comparison of proposed method 1, proposed method 2 and constnt plus proportional rate reaching law

From the above table 2   gives the comparison analysis of the proposed method1, method 2  and consnt plus propotional rate reaching law.The reaching time to steady state by the method 1 takes little bit more than the method 2,The output voltage of  sigmoid reaching law is more  than the tanhyperbolic reaching law(R-Load).The output voltage of R-C load of sigmoid reaching law is more than the tanhyperbolic reaching law ,similarly the output voltage of  R-L-C load of sigmoid reaching law is more than the tanhyperbolic reaching law. Proposed method 2 exhibits the dyamic and less insensitive to load parameter varaitions.The sigmoid reaching  law output voltage is   away from desired output voltage from all loading conditions.Whereas the tanhyperbolioc reaching law is nearer to the desired output voltage and less ripple in the output voltage. This is due to chattering in the sigmoid reaching law, intun more deviated output voltage from the desired output voltage.With this tanhyperbolic reaching law gives better resuts than the sigmoid reaching law and constant plus proportional rate reaching law.

CONCLUSION

The proposed method 2 gives constant output voltage, reaches steady state with fast convergence time and it mitigates chattering, it covers entire sliding mode portion of the phase plane trajectory as compared to method 1. The different loading  are applied to converter ,among these two methods  the tanhyperbolic reaching law gives better controlling the output voltage and less ripple in the output voltage than the sigmoid reaching law and consnt pls protionalrate reaching law. Tanhyperbolic reaching law gives less chattering and takes less time to reach the steady state.In turn less switching losses in the dc-dc buck converter.

REFERENCES

- Weibing Gao, and James C. Hung, “Variable Structure Control of Nonlinear Systems:A New Approach” IEEE  Transactions on Industrial Electronics, vol.40, no.1, pp. 1234-1241. Feb.1993.

- John Y.Hung, Weibing Gao, and James C. Hung, “Variable Structure Control:A Survey” IEEE  Transactions on Industrial Electronics, Vol. 40, No.1,pp.239-246, Feb. 1993

- Y. He and F.L.Luo “Design and Analysis of Adaptive Sliding-Mode-Like Controller for DC–DC converter” The Institution of Engineering and Technology.IEE Proceedings online no. 20050222. IEE Proc.-Electr.Power Appl.Vol. 153, No. 3, pp-125-132, May 2006.

- Siew-Chong Tan, Y. M. Lai, , Chi K. Tse and  Martin K. H. Cheung, “A Fixed-Frequency Pulse width  Modulation

Based Quasi-Sliding-Mode Controller for Buck Converter” IEEE Transactions on Power Electronics,   Vol. 20,

No. 6, pp-1234-1241,Nov.2005.

- Shrey Kasera,Amit Kumar and L. B. Prasad “Analysis of chattering free improved sliding mode control” .Innovations and Information Embedded and Communication Systems (ICIIECS), 2017 International Conference.  Date of Conference Pages.  617-622, 18-20th  March 2017.Coimbatore, India.

- Pranav ,J.Kumar,V. Kumar,K.P and S. Rana  “Efficient Reaching Law for SMC with PID Surface    Applied to a Manipulator” 978-1-4673-9080-4/16/$31.00 ©2016 IEEE

- Peng Zhang, andQingxian Wu”Double power reaching law for sliding mode control with unidirectional auxiliary surfaces”. Chinese Control Conference (CCC), 2017 36th  Chinese,26-28th July 2017, Pages 3798 - 3803, Dalian, China.

[8]     Quanlong HU, Ling LIU.Hang YANG, “A sliding mode speed controller based on novel reaching law

of permanent magnet synchronous motor system” 978-1-5386-3524-7/17/$31.00 ©2017 IEEE.

[9]    Liang Tao, Qiang Chen, Yurong Nan and Chun W,” Double Hyperbolic Reaching Law With Chattering-Free and

Fast  convergence”. IEEE Access Multidisciplinary journal digital Object Dentifier

0.1109/ACCESS.2017.DOI.2018.

[10]   Yun-long Liu, Cun-chen Gao, and Xiao Liang“Variable Structure Control for Affine Systems Based onSigmoid

Variable Rate Reaching Law’’Proceedings of the 8th World Congress on Intelligent Control and Automation July

6- 9th  2010, Jinan, China.

[11]   H.N.Nagaraja,Rajanna.G.S  “Comparison between sigmoid variable reaching law and exponential reaching law for

# sliding mode controlled DC-DC Buck converter” Published in: 2013. International Conference on Power, Energy   and Control (ICPEC), Date Added to IEEE Xplore: 13 June 2013, DOI: 10.1109/ICPEC.2013.6527673.

# [12]  Yazan M. Alsmadi, Vadim Utkin, Mohammed A. Haj-ahmed & Longya Xu Sliding mode control of power

Converters: DC/DC Converters International Journal of Control, ISSN: 0020-7179 (Print) 1366-5820 (Online)

Journal  2017

[13]  K. B. Devika and Susy Thomas “Improved sliding mode controller performance through power rate exponential

reaching law” Second International Conference on Electrical, Computer and Communication Technologies     .                             (ICECCT)22-24th  Feb. 2017 , No.2.Pages 1 – 7, 23rd  November 2017 Coimbatore,India

[14]  Hongtao Ye,Mengmeng Li and Wenguang Luo, “A novel reaching law of sliding mode control design and analysis”

20-22nd  Oct. 2017,  Pages 1803 – 1807, 2017 Chinese Automation Congress (CAC). Jinan, China.

[15]  Yongheng Li, Lingshun Liu ,”The Research of the Sliding Mode Control Method Based on Improved Double

Reaching Law”978-1-5386-1243-9/18/$31.00_c 2018 IEEE

[16]  Honggui Han ,  Xiaolong Wu , and Junfei Qiao, ” Design of  Robust Sliding Mode Control With Adaptive

Reaching Law” IEEE Transactions On Systems, Man, And Cybernetics: Systems, Digital  Object Identifier

10.1109/TSMC.2018.2852626, accepted June 28, 2018.

[17]  Haifeng Ma , Yangmin Li ,  and Zhenhua Xiong ” Discrete-Time Sliding-Mode Control With Enhanced  Power

Reaching Law” IEEE Transactions On Industrial Electronics, Vol. 66, No. 6, June 2019

[18]  Jagannath Samantaray and Sohom Chakrabarty, “Digital Implementation of Sliding Mode Controllers with DC-DC

Buck Converter System” 2018 15th International Workshop on Variable Structure Systems (VSS) Graz University

Of Technology, Graz, Austria, July 9-11, 2018

[19]  Yiwen Hey, Weisheng Xu, and Yan Cheng “A Novel Scheme for Sliding-Mode Control of DC-DC Buck onverters

with a Constant Frequency Based on the Averaging Mode” Journal of Power Electronics, Vol. 10,  No. 1, January

2010, JPE 10-1-1

[20]   Jianweni leng and Chang Ma, “Slidng mode control  for PMSM based on a novel hybrid reaching law”,Proceeding

27 the Chinese control conference, July25-27, 2018 Wuhan,China

BIOGRAPHIES OF AUTHORS (10 PT)

|  | K.B. Siddesh is Research Scholar at VTU, Karnataka, India, and currently working as an Associate Professor in the Department of Electronics and Communication Engineering at SJMIT, Chitradurga, India. His area of interest includes Power Electronics & Drives, Insulation Coordination and Power Quality Issues. He has 20 Years of Teaching Experience. |
| --- | --- |
|  | Dr. Basavaraja Banakara is currently working as a Professor in the Department of Electrical and Electronics Engineering at University of BDT Engineering College (Constituent College of Visvesvaraya Technological University) Davanagere, Karnataka,India. He has 26 Years of Teaching Experience, 13 Years of Research & Developments and 15 Years of Administration. His area of interest includes Power Electronics and Drives, Insulation Coordination and Power Quality Issues. He has published more number of papers in various International and National Journals |
|  | R.Shivarudraswamy is currently working as a Assoc.professor in EEE Department, Manipal institute of Technology,Manipal .His area of interests are Insulation Coordination and Power Quality Issues. He has 21 Years of Teaching Experience. |