# Artificial intelligence approach to laboratory prediction energy control system

Desmira1,2, Norazhar Abu Bakar1, Mohd Ruzaini Hashim1, Romi Wiryadinata3, Mustofa Abi Hamid2

1Department of Electrical Engineering, Faculty of Electrical Engineering, Universiti Teknikal Malaysia Melaka (UTeM), Malaysia

2Department of Electrical Engineering Vocational Education, Faculty of Teacher Training and Education, Universitas Sultan Ageng Tirtayasa, Serang, Indonesia

3Department of Electrical Engineering, Faculty of Engineering, Universitas Sultan Ageng Tirtayasa, Serang, Indonesia

| Article Info |  | ABSTRACT |
| --- | --- | --- |
| Article history: Received month dd, yyyy Revised month dd, yyyy Accepted month dd, yyyy |  | The use of electrical energy is increasing every year in all areas of life. Laboratory Prediction Energy Control System (LPECS) was to predict energy demand. This research conducted in the Electrical Engineering Vocational Education laboratory by comparing ANFIS (Artificial Neural Fuzzy System) with Fuzzy logic. The comparison of methods aimed to determine the reliability of the two ways in the energy demand prediction system. The results showed that the minimum value of the target data using the conventional method (actual data) was 44,42%, with the prediction data using the ANFIS method was 44.33%, and the prediction data using the Fuzzy method being 59,31%. The maximum value of the conventional way (actual data) between ANFIS and Fuzzy had the same value, namely 77,59%. Then, the RMSE ANFIS value was 0,1355%, the Mean Absolute Percentage Error (MAPE) was 0,2791%, and Fuzzy logic was 0,1986%. Thus, ANFIS can be used to determine the minimum and maximum values. Meanwhile, fuzzy can only show the maximum value but cannot reach the minimum value properly. |
| Keywords: Artificial intelligence prediction energy control ANFIS Fuzzy logic |  |  |
|  |  | This is an open access article under the CC BY-SA license. |
| Corresponding Author: Norazhar Abu Bakar Department of Electrical Engineering, Faculty of Electrical Engineering, Universiti Teknikal Malaysia Melaka (UTeM) Hang Tuah Jaya, Durian Tunggal, Melaka 76100, Malaysia Email: norazhar@utem.edu.my |  |  |

- INTRODUCTION

The need for energy is increasing day by day. Save energy use in the room has various efforts. With artificial intelligence (AI), the energy need in a room will be easy to control. The use of energy in a building or building makes a very positive contribution in evaluating energy use with the AI approach [1] [2]. Energy requirements in a room have been widely discussed including using the ANFIS algorithm [3] [4], Fuzzy Logic [5] [6] [7] [8] [9], ANN [10] [11] [12] [13] [14] [15] [16] [17] [18] and GA [19] [20]. Several studies that discuss the energy consumption of a building have been carried out, including comparing Artificial Neural Network (ANN) algorithms, clustering, statical and machine learning, and Support Vector Machine (SVM). The SVM algorithm showed the best results in predicting energy consumption in a building, but it has not explained how much error accuracy was contained in each algorithm used [14]. The building of a school in Kuopio Eastern Finland researched energy consumption between electricity and heating [20].

This study presents a computational model to estimate energy consumption in the Electrical Engineering Vocational Education (EEVE) laboratory using Artificial Intelligence (AI). The FCM was a clustering technique for the ANFIS model, where similar time series data are used due to its accuracy and lower computational time [21]. The performance of the inference system adapted to the Adaptive Neuro-Fuzzy Inference System (ANFIS), and it is hybrid with Particle Swarm Optimization (PSO) performance in predicting energy consumption from climate factors for multi-campus institutions in South Africa [22]. Changes in energy consumption forecasting systems used fuzzy logic to reduce uncertainty, inconvenience, and inefficiency [5] gross annual electricity demand for short-term Turkey by applying the fuzzy logic methodology [6]. Four ANFIS models, developed, trained, and validated with a trial performance data set that collected and applied to predict the operating temperature of the system [23]. In this study, an Adaptive Neuro-Fuzzy Inference System (ANFIS) was designed and adapted to estimate the energy consumption of the building according to the main building envelope parameters, namely the thickness of the material and the value of K insulation [24]. The results showed that ANN and ANFIS predicted the energy consumption of the cooling load of the three buildings with reasonable accuracy. The correlation coefficient between measured and predicted consumption for the training data was well above 0,98. The same was well above 0,96 for the test data [25]. This paper presented and evaluated control strategies for adjusting and preserving air quality, thermal and visual comfort for building occupants. At the same time, simultaneously, energy consumption reduc-tion is achieved [26], managing energy in residential buildings effectively, an efficient energy control system is required, which can reduce total energy consumption without compromising the user's preferred environment in the building [27]. Gradient-based optimization can be used to minimize the energy consumption of distributed environmental control systems without increasing occupant thermal dissatisfaction [28].

High-resolution household electricity usage model using Fuzzy Logic Inference System. Using the input pattern of active occupancy and typical household habits, the fuzzy model provides the possibility to start each piece of equipment within the next minute as the output [29] of the designed controller and significant lighting energy can be saved. Offices, where smart LED lighting systems are installed can automatically adjust lighting output based on user movement and allow users to select their lighting preferences [30]. The fuzzy logic controller can be applied to EAHX to reduce more electrical energy consumption. A simulation is carried out, where the thermodynamic model of EAHX is obtained and used together with fuzzy logic controller simulation, then compared with On-Off controller simulation [31] control scheme FIS and ANN to simultaneously control the amount of air supply and temperature. The study's conclusion is that mass and temperature control simultaneously maintain the desired room temperature in a very efficient manner [32]. Neural networks and fuzzy systems have some similarities. If there is no mathematical model of the problem given, both can be used to solve the problem [33]. The method that combines the two techniques is commonly called a hybrid system, one of which is the Adaptive Neuro-Fuzzy Inference System or ANFIS [34]. The input-output data set of the HVAC system is first stored, and the data set for predicting fan motor speed is based on an Adaptive Network-Based Fuzzy Inference System (ANFIS). In the simulation, the root-mean-square (RMS) and the coefficient of multiple determination (R2) as two performance measures were obtained to compare the predicted value and the actual value of the model validation, the results obtained in the statistical analysis obtained an RMS value of 3,3475 and an R2 value for evaporator 0,9954 while the RMS value was 15,6750 and R2 was 0,9402 for zone-1 and RMS was 17,7019 and R2 was 0,9410 for zone-2 for the ANFIS model [35]. Predicted Quality of Experience (QoE) in a student-centered mixed learning environment, equipped with technologically enriched classrooms, the model used ANFIS seven and four input variables and compared their performance using RMSE, MAPE, and R2 measurements. The results showed that perceived QoE could be reliably predicted by personality traits and student learning styles as subjective factors and network filters as objective factors [36].

A laboratory Prediction Energy Control System (LPECS) used the ANFIS method and Fuzzy Logic. Laboratory Prediction Energy Control System (LPECS) was compared with conventional mea-surements (actual data) and artificial intelligence (AI). The algorithm used in this study was to compare the real (traditional) data measurement to ANFIS and fuzzy. It was predicting humidity in a laboratory. The study that discussed applying fuzzy logic in a manufacturer to predict energy consumption showed the percentage change in factory energy consumption after analyzing the three input parameters. These findings can provide a solid basis on which decision-makers and systems analysts can base appropriate strategies to ensure the efficiency and stability of the manufacturing system [5]. The proposed technique was evaluated using FIS Mamdani and FIS Sugeno. The proposed method provided a flexible and energy-efficient decision-making system that maintains users' thermal comfort with the help of intelligent sensors [37]. This stimulus model was constructed into fuzzy Mamdani type based on modeling rules (RBMTF), using input parameters (Uw, Te) and output parameters d2, described by the if-then rule RBMTF the test data about 97,4%. Overall, RBMTF can be used as a reliable modeling method for the thermal performance of laminated precast concrete panels used in residential building studies [38]. Fuzzy with Mamdani fuzzy inference method was mainly applied only at the simulation level in this study comparing MATLAB simulation with microcontroller programming. In programming, the microcontroller used a methodology that can produce the same simulation results [39][40].

1.1.  Fuzzy Inference System

ANFIS (Adaptive Neuro-Fuzzy Inference System), applied to LPECS, was an artificial neural network that involves the Takagi–Sugeno–Kang (TSK) model of fuzzy inference system. The ANFIS used consists of one input layer, three hidden layers, and one output layer. The neuron represented by a square shape was a parameter of the TSK fuzzy membership function; LPECS Fuzzy Inference System (FIS) was a computational framework based on ANFIS theory consisting of 5 (five) parts in Figure 1 [24].

Figure 1. ANFIS Architecture

Layer 1 (fuzzification): each neuron was adaptive to the parameters of an activation. The output of each neuron was the degree of membership given by the membership function. The input membership function used a trapezoidal membership type (trapmf).

Yi =  µi (xi)……………………………..……………………………..…………………………… (1)

Layer 2 (Rules): this layer was a fixed neuron (given the symbol П) which was the product of all inputs, as in equation 2.

Wi =   μAi(xi)μBi(x2) ……………………………………………………………………………… (2)

The AND operator was usually used. The result of this calculation is called the firing strength of a rule. Each neuron represents the i-rule.

Layer 3 (Normalization): each neuron in this layer was a fixed neuron (given the symbol N) which was the result of the calculation of the ratio of the-i firing strength (wi) to the sum of the overall firing strength in the second layer, as in equation 3.

=   wii=1nwi  ………………………………………………………………………………………. (3)

Layer 4 (Defuzzification): this layer was in the form of neurons which were adaptive neurons to an output, as in equation 4.

.fi = (piZit+qiZ2tr1)… ……………………………………………………………………..(4)

Layer 5 (Network Output): this layer was a single neuron (symbolized) resulting from the sum of all the outputs of the fourth layer, as in question 5.

Y =  i=1nwifi   …………………...……………………...….…………………...…..…………….. (5)

Research data obtained for approximately three months were taken into input data and output data. The input data was in the form of power (X1) and temperature (X2), while the output data was in the form of humidity (Xt).

1.2.  Prediction Performance ANFIS Test and Fuzzy Logic

There were several ways to test and measure the performance of LPECS prediction comparisons between ANFIS and Fuzzy Logic. Among them were Root Mean Square Error (RMSE) and Mean Absolute Percentage Error (MAPE). RSME was used to find the accuracy of forecasting results with target data, namely humidity data taken conventionally in the Electrical Engineering Vocational Education (EEVE) laboratory. The smaller the humidity forecast value with target or goal data in the form of humidity. The result of forecasting results was carried out, and the RSME value starts from 0 to infinity, with 0 being the best value. RMSE can be calculated by equation 6.

RMSE=1nyi-yt 2n  ………………………………………...…………………………………… (6)

The next LPECS performance measurement used MAPE, namely calculating the difference between conventional (actual) data and forecasting data using the ANFIS and Fuzzy algorithms. The difference was calculated in the form of a percentage of the actual (conventional) data. The subsequent LPECS performance measurement used MAPE, namely calculating the difference between the original and forecasted data. The difference was entered according to the MAPE formula and then calculated as a percentage of the original data.

MAPE=yi-yt 2yin x 100 ………………………………………………………...……………… (7)

- METHOD

Figure 2 described the research flow starting with data collection as follows:

Stage 1 ANFIS

Stage 2 Fuzzy

Parameter Input

Power

Temperature

Ouput

Humidity accuracy of forecasting results with RMSE and MAPE

Figure 2. LPECS Research

Figure 2 parameters used in the form of power, temperature, and humidity data output target. After the data was collected, it enters the fuzzification step, where the information was grouped into input and output data according to the ANFIS steps equation (1). Furthermore, the data was further divided into training data, testing data, and demo data. The following process was to create a rule according to equation (2). The data was then normalized in the range [0 1], then the information was entered in equation (3). In stage 1, the data used are training data and testing data. The following process is to form a generate FIS. This layer defuzzification was in the form of adaptive neurons to output with equation (4). The membership function value was determined in this process, the type of membership function used, and the desired result. Then enter the FIS training stage and determine the epoch value and the method used (hybrid and backpropagation). From equation (5), the steps for ANFIS were to add up all the input variables from the process of equation (4). The result will be the average RMSE value according to equation (6). After being trained, enter the FIS testing stage. Just like the FIS training in the data testing process, the average RMSE value will be obtained. If the RMSE value was high enough, it will return to the FIS generate process, but if the RMSE value was good enough, it will go to the next stage, predicting and comparing actual (conventional) data with fuzzy. Predictive data using the Adaptive Neuro-Fuzzy Inference System (ANFIS) algorithm were regrouped. Finally, the comparison value of actual (conventional) data with predictive data was obtained using the Adaptive Neuro-Fuzzy Inference System algorithm.

From equation (5), the steps for ANFIS were to add up all the input variables from the process of equation (4). The result will be the average RMSE value according to equation (6). After being trained, enter the FIS testing stage. Just like the FIS training in the data testing process, the average RMSE value will be obtained. If the RMSE value is high enough, it will return to the FIS generate process, but if the RMSE value is good enough, it will go to the next stage, predicting and comparing actual (conventional) data with fuzzy. Predictive data using the Adaptive Neuro-Fuzzy Inference System (ANFIS) algorithm were regrouped. Finally, the comparison value of actual (conventional) data with predictive data was obtained using the Adaptive Neuro-Fuzzy Inference System algorithm.

- RESULTS AND DISCUSSION

From the results of the study using the Adaptive Neuro Fuzzy Inference System algorithm, it can be used as a measurement prediction for actual (conventional) data by producing a fairly good average Roat Mean Square Error (RMSE) value. The RMSE results for each type of membership are shown in Table 1.

Table 1. Comparison of RMSE Value of Each Membership Type

| No | Membership Type | Hybrid |  | Backpropagation |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Training Data | Testing Data | Training Data | Testing Data |
| 1. | Trimf | 0,1284 | 0,2881 | 0,1516 | 0,2934 |
| 2. | Trapmf | 0,1355 | 0,2791 | 0,1523 | 0,2834 |
| 3. | Gbellmf | 0,1118 | 0,7413 | 0,1627 | 0,2799 |
| 4. | Gaussmf | 0,1106 | 0,3923 | 0,1657 | 0,2838 |

From Figure 1 the parameters used are power and temperature and the desired target data is humidity. After the data is collected, it enters the fuzzification step where the data is grouped into input data and output data according to the ANFIS steps equation (1). Furthermore, the data is further divided into training data, testing data, and demo data. The next process is to create an appropriate rule.

Figure 3. Comparison Graph of LPECS Actual Data (Conventional Data), ANFIS, and Fuzzy against APE

Figure 3 shows that the Adaptive Neuro Fuzzy Inference System (ANFIS), which is a combination of artificial neural networks and fuzzy logic, is the best method for making predictions. This can be seen from the minimum value of actual data (conventional data) before being tested, which is 44.42% after being tested using the ANFIS method to 44.33%. If the results are compared with fuzzy, it will be much different, which is 59.31%. At the maximum value, the actual data (conventional data) has the same value as ANFIS and also fuzzy, which is 77.59%. ANFIS can be used well as a determinant of the minimum and maximum values, while fuzzy can only show the maximum value but cannot reach the minimum value properly.The best type of membership is the type of membership that produces the lowest RMSE value. Based on table 1, we can see that the best RMSE value is using the trapezoidal membership type (trapmf). The LPECS comparison graph between the actual data (conventional against the Adaptive Neuro Fuzzy Inference System (ANFIS) and fuzzy). The blue color on the graph shows the humidity data taken conventionally in the Electrical Engineering Vocational Education (EEVE) laboratory, while the red color uses the ANFIS algorithm, the yellow color shows the data that is processed using the fuzzy algorithm. The target data for the output test is humidity data data

The following is a trapezoidal membership type data traning (trapmf). The output uses a linear type membership function (MF). The results of the training data using the trapezoidal hybrid method (trapmf) obtained an RMSE value of 0.1355 as shown in Figure 5.

Figure 4 FIS Train Using a trapezium (Trapmf)

Furthermore, the FIS Test uses the hybrid method in Figure 5 for trapezoidal membership types (trapmf). The results obtained an average RMSE value of 0.2791%.

Figure 5 FIS Testing Using a Trapezoid (Trapmf)

The author compares the Adaptive Neuro Fuzzy Inference System (ANFIS) method using the Fuzzy method to actual data (conventional data). The results of the actual (conventional) humidity output value, the ANFIS method, and the fuzzy can be seen in table 2.

Table 2 Comparison of LPECS values for Actual Data (Prediction Data), ANFIS, Fuzzy Error (Matlab) and APE

Table 2. Comparison of LPECS values for Actual Data ( Prediction Data ), ANFIS, Fuzzy Error ( Matlab ) and APE

| No | Data Aktual | ANFIS | Fuzzy | Errors (Matlab) | APE |
| --- | --- | --- | --- | --- | --- |
| 1 | 46.33 | 46.44 | 59.31 | 12.98 | 0.2801 |
| 2 | 50.49 | 50.5 | 60.45 | 9.96 | 0.1972 |
| 3 | 57.66 | 57.08 | 69.6 | 11.94 | 0.2070 |
| 4 | 62.23 | 62.23 | 64.46 | 2.23 | 0.0358 |
| 5 | 56.99 | 57.02 | 71.86 | 14.87 | 0.2609 |
| 6 | 60.79 | 60.75 | 64.46 | 3.67 | 0.0603 |
| 7 | 77.59 | 77.59 | 77.59 | 0 | 0 |
| 8 | 45.41 | 45.32 | 60.45 | 15.04 | 0.3312 |
| 9 | 51.91 | 50.71 | 67.3 | 15.39 | 0.2964 |
| 10 | 47.23 | 48.61 | 69.02 | 21.79 | 0.4613 |
| 11 | 45.26 | 45.38 | 62.74 | 17.48 | 0.3862 |
| 12 | 74.14 | 74.16 | 77.59 | 3.45 | 0.0465 |
| 13 | 44.42 | 44.33 | 60.45 | 16.03 | 0.3608 |
| 14 | 53.21 | 53.24 | 63.87 | 10.66 | 0.2003 |
| 15 | 53.69 | 53.59 | 60.45 | 6.76 | 0.1259 |
| 16 | 68.84 | 68.67 | 71.31 | 2.47 | 0.0358 |
| 17 | 61.26 | 61.13 | 66.72 | 5.46 | 0.0891 |
| MAPE |  |  |  |  | 0.1985 |

The results of data processing using fuzzy logic show an average result that has a higher value height can be seen from table 2 . Each humidity value in the actual (conventional), ANFIS, and Fuzzy data is 56.32; 66.33 ; 56.28. As for the comparison graph of actual (conventional), ANFIS, and Fuzzy data values, it can be seen in Figure .

All nodes in this layer are non-adaptive as a result of the predictions made and the value of the RMSE starts from 0 to infinity, with 0 being the best value. RSME can be calculated by equation (6).

Table 2 can be seen to compare the actual data (conventional data) Fuzzy, Error (Matlab) and APE. The results compared to fuzzy will be much different, which is 59.31%. To obtain MAPE using equation (7) the results of processing using MAPE fuzzy logic 0.1985%.

- CONCLUSION

From the results obtained from research in the laboratory Prediction Energy Control System (LPECS) predicts energy needs. This can be seen from the minimum value of actual data (conventional data) before processing, which is 44.42% after being processed using the ANFIS method to 44.33%. If the results are compared with fuzzy, it will be much different, which is 59.31%. At the maximum value, the actual data has the same value as ANFIS and also fuzzy, which is 77.59%. ANFIS can be used well as a minimum and maximum value prediction, while fuzzy can only show the maximum value but cannot reach the minimum value properly.

To predict the need for electrical energy consumption in the electrical engineering vocational education laboratory (PVTE) using the ANFIS algorithm and 0.1355% Fuzzy Logic is 0.2791% for ANFIS RMSE and 0.1985% Fuzzy Logic MAPE values. So, it can be concluded that the error value between ANFIS and Fuzzy Logic is almost the same, namely 0.1. From the conclusion to predict the use of electrical energy in a room can use the ANFIS algorithm and fuzzy logic can be applied properly.

REFERENCES

[1]	Z. Wang and R. S. Srinivasan, “A review of artificial intelligence based building energy prediction with a focus on ensemble prediction models,” Proceedings - Winter Simulation Conference, vol. 2016-February, pp. 3438–3448, Feb. 2016, doi: 10.1109/WSC.2015.7408504.

[2]	S. S. K. Kwok, R. K. K. Yuen, and E. W. M. Lee, “An intelligent approach to assessing the effect of building occupancy on building cooling load prediction,” Building and Environment, vol. 46, no. 8, pp. 1681–1690, Aug. 2011, doi: 10.1016/J.BUILDENV.2011.02.008.

[3]	B. Bektas Ekici and U. T. Aksoy, “Prediction of building energy needs in early stage of design by using ANFIS,” Expert Systems with Applications, vol. 38, no. 5, pp. 5352–5358, May 2011, doi: 10.1016/J.ESWA.2010.10.021.

[4]	Q. Xie, J. Q. Ni, J. Bao, and Z. Su, “A thermal environmental model for indoor air temperature prediction and energy consumption in pig building,” Building and Environment, vol. 161, p. 106238, Aug. 2019, doi: 10.1016/J.BUILDENV.2019.106238.

[5]	H. C. W. Lau, E. N. M. Cheng, C. K. M. Lee, and G. T. S. Ho, “A fuzzy logic approach to forecast energy consumption change in a manufacturing system,” Expert Systems with Applications, vol. 34, no. 3, pp. 1813–1824, Apr. 2008, doi: 10.1016/J.ESWA.2007.02.015.

[6]	S. Kucukali and K. Baris, “Turkey’s short-term gross annual electricity demand forecast by fuzzy logic approach,” Energy Policy, vol. 38, no. 5, pp. 2438–2445, May 2010, doi: 10.1016/J.ENPOL.2009.12.037.

[7]	S. Bissey, S. Jacques, and J. C. Le Bunetel, “The Fuzzy Logic Method to Efficiently Optimize Electricity Consumption in Individual Housing,” Energies 2017, Vol. 10, Page 1701, vol. 10, no. 11, p. 1701, Oct. 2017, doi: 10.3390/EN10111701.

[8]	D. Desmira, M. A. Hamid, Irwanto, S. D. Ramdani, and T. Y. Pratama, “An ultrasonic and temperature sensor prototype using fuzzy method for guiding blind people,” Journal of Physics: Conference Series, vol. 1446, no. 1, p. 012045, Jan. 2020, doi: 10.1088/1742-6596/1446/1/012045.

[9]	D. Desmira, M. A. Hamid, N. A. Bakar, M. Nurtanto, and S. Sunardi, “Smart Traffic Lights Using Microcontroller Based on Fuzzy Logic,” IAES International Journal of Artificial Intelligence, vol. 11, no. 1, 2022.

[10]	C. Deb, L. S. Eang, J. Yang, and M. Santamouris, “Forecasting diurnal cooling energy load for institutional buildings using Artificial Neural Networks,” Energy and Buildings, vol. 121, pp. 284–297, Jun. 2016, doi: 10.1016/J.ENBUILD.2015.12.050.

[11]	A. H. Neto and F. A. S. Fiorelli, “Comparison between detailed model simulation and artificial neural network for forecasting building energy consumption,” Energy and Buildings, vol. 40, no. 12, pp. 2169–2176, Jan. 2008, doi: 10.1016/J.ENBUILD.2008.06.013.

[12]	M. Kim, S. Jung, and J. W. Kang, “Artificial Neural Network-Based Residential Energy Consumption Prediction Models Considering Residential Building Information and User Features in South Korea,” Sustainability 2020, Vol. 12, Page 109, vol. 12, no. 1, p. 109, Dec. 2019, doi: 10.3390/SU12010109.

[13]	S. R. Mohandes, X. Zhang, and A. Mahdiyar, “A comprehensive review on the application of artificial neural networks in building energy analysis,” Neurocomputing, vol. 340, pp. 55–75, May 2019, doi: 10.1016/J.NEUCOM.2019.02.040.

[14]	T. Ahmad, H. Chen, Y. Guo, and J. Wang, “A comprehensive overview on the data driven and large scale based approaches for forecasting of building energy demand: A review,” Energy and Buildings, vol. 165, pp. 301–320, Apr. 2018, doi: 10.1016/J.ENBUILD.2018.01.017.

[15]	K. Li, H. Su, and J. Chu, “Forecasting building energy consumption using neural networks and hybrid neuro-fuzzy system: A comparative study,” Energy and Buildings, vol. 43, no. 10, pp. 2893–2899, Oct. 2011, doi: 10.1016/J.ENBUILD.2011.07.010.

[16]	J. S. Chou and D. K. Bui, “Modeling heating and cooling loads by artificial intelligence for energy-efficient building design,” Energy and Buildings, vol. 82, pp. 437–446, Oct. 2014, doi: 10.1016/J.ENBUILD.2014.07.036.

[17]	R. Mena, F. Rodríguez, M. Castilla, and M. R. Arahal, “A prediction model based on neural networks for the energy consumption of a bioclimatic building,” Energy and Buildings, vol. 82, pp. 142–155, Oct. 2014, doi: 10.1016/J.ENBUILD.2014.06.052.

[18]	A. Azadeh, S. F. Ghaderi, S. Tarverdian, and M. Saberi, “Integration of artificial neural networks and genetic algorithm to predict electrical energy consumption,” Applied Mathematics and Computation, vol. 186, no. 2, pp. 1731–1741, Mar. 2007, doi: 10.1016/J.AMC.2006.08.093.

[19]	H. Ceylan and H. K. Ozturk, “Estimating energy demand of Turkey based on economic indicators using genetic algorithm approach,” Energy Conversion and Management, vol. 45, no. 15–16, pp. 2525–2537, Sep. 2004, doi: 10.1016/J.ENCONMAN.2003.11.010.

[20]	M. Raatikainen, J. P. Skön, K. Leiviskä, and M. Kolehmainen, “Intelligent analysis of energy consumption in school buildings,” Applied Energy, vol. 165, pp. 416–429, Mar. 2016, doi: 10.1016/J.APENERGY.2015.12.072.

[21]	P. Adedeji, N. Madushele, and S. Akinlabi, “Adaptive Neuro-fuzzy Inference System (ANFIS) for a multi-campus institution energy consumption forecast in South Africa,” in Proceedings of the International Conference on Industrial Engineering and Operations Management, Sep. 2018, pp. 950–958.

[22]	P. A. Adedeji, S. Akinlabi, N. Madushele, and O. O. Olatunji, “Hybrid adaptive neuro-fuzzy inference system (ANFIS) for a multi-campus university energy consumption forecast,” https://doi.org/10.1080/01430750.2020.1719885, 2020, doi: 10.1080/01430750.2020.1719885.

[23]	L. Yang and E. Entchev, “Performance prediction of a hybrid microgeneration system using Adaptive Neuro-Fuzzy Inference System (ANFIS) technique,” Applied Energy, vol. 134, pp. 197–203, Dec. 2014, doi: 10.1016/J.APENERGY.2014.08.022.

[24]	S. Naji et al., “Application of adaptive neuro-fuzzy methodology for estimating building energy consumption,” Renewable and Sustainable Energy Reviews, vol. 53, pp. 1520–1528, Jan. 2016, doi: 10.1016/J.RSER.2015.09.062.

[25]	C. Deb, L. S. Eang, J. Yang, and M. Santamouris, “Forecasting Energy Consumption of Institutional Buildings in Singapore,” Procedia Engineering, vol. 121, pp. 1734–1740, Jan. 2015, doi: 10.1016/J.PROENG.2015.09.144.

[26]	D. Kolokotsa, D. Tsiavos, G. S. Stavrakakis, K. Kalaitzakis, and E. Antonidakis, “Advanced fuzzy logic controllers design and evaluation for buildings’ occupants thermal–visual comfort and indoor air quality satisfaction,” Energy and Buildings, vol. 33, no. 6, pp. 531–543, Jul. 2001, doi: 10.1016/S0378-7788(00)00098-0.

[27]	M. Fayaz and D. H. Kim, “Energy Consumption Optimization and User Comfort Management in Residential Buildings Using a Bat Algorithm and Fuzzy Logic,” Energies 2018, Vol. 11, Page 161, vol. 11, no. 1, p. 161, Jan. 2018, doi: 10.3390/EN11010161.

[28]	S. Ari, I. A. Cosden, H. E. Khalifa, J. F. Dannenhoffer, P. Wilcoxen, and C. Isik, “Constrained fuzzy logic approximation for indoor comfort and energy optimization,” Annual Conference of the North American Fuzzy Information Processing Society - NAFIPS, vol. 2005, pp. 500–504, 2005, doi: 10.1109/NAFIPS.2005.1548586.

[29]	L. Ciabattoni, M. Grisostomi, G. Ippoliti, and S. Longhi, “Fuzzy logic home energy consumption modeling for residential photovoltaic plant sizing in the new Italian scenario,” Energy, vol. 74, no. 1, pp. 359–367, Sep. 2014, doi: 10.1016/J.ENERGY.2014.06.100.

[30]	J. Liu, W. Zhang, X. Chu, and Y. Liu, “Fuzzy logic controller for energy savings in a smart LED lighting system considering lighting comfort and daylight,” Energy and Buildings, vol. 127, pp. 95–104, Sep. 2016, doi: 10.1016/J.ENBUILD.2016.05.066.

[31]	S. E. Diaz, J. M. T. Sierra, and J. A. Herrera, “The use of earth–air heat exchanger and fuzzy logic control can reduce energy consumption and environmental concerns even more,” Energy and Buildings, vol. 65, pp. 458–463, Oct. 2013, doi: 10.1016/J.ENBUILD.2013.06.028.

[32]	J. Ahn, S. Cho, and D. H. Chung, “Analysis of energy and control efficiencies of fuzzy logic and artificial neural network technologies in the heating energy supply system responding to the changes of user demands,” Applied Energy, vol. 190, pp. 222–231, Mar. 2017, doi: 10.1016/J.APENERGY.2016.12.155.

[33]	J. J. Buckley and H. Yoichi, “Neural nets for fuzzy systems,” Fuzzy Sets and Systems, vol. 71, no. 3, pp. 265–276, May 1995, doi: 10.1016/0165-0114(94)00282-C.

[34]	J. S. R. Jang, “ANFIS: Adaptive-Network-Based Fuzzy Inference System,” IEEE Transactions on Systems, Man and Cybernetics, vol. 23, no. 3, pp. 665–685, 1993, doi: 10.1109/21.256541.

[35]	S. Soyguder and H. Alli, “Predicting of fan speed for energy saving in HVAC system based on adaptive network based fuzzy inference system,” Expert Systems with Applications, vol. 36, no. 4, pp. 8631–8638, May 2009, doi: 10.1016/J.ESWA.2008.10.033.

[36]	T. Vasileva-Stojanovska, M. Vasileva, T. Malinovski, and V. Trajkovik, “An ANFIS model of quality of experience prediction in education,” Applied Soft Computing, vol. 34, pp. 129–138, Sep. 2015, doi: 10.1016/J.ASOC.2015.04.047.

[37]	Qurat-Ul-Ain, S. Iqbal, S. A. Khan, A. W. Malik, I. Ahmad, and N. Javaid, “IoT Operating System Based Fuzzy Inference System for Home Energy Management System in Smart Buildings,” Sensors 2018, Vol. 18, Page 2802, vol. 18, no. 9, p. 2802, Aug. 2018, doi: 10.3390/S18092802.

[38]	M. Tosun, K. Dincer, and S. Baskaya, “Rule-based Mamdani-type fuzzy modelling of thermal performance of multi-layer precast concrete panels used in residential buildings in Turkey,” Expert Systems with Applications, vol. 38, no. 5, pp. 5553–5560, May 2011, doi: 10.1016/J.ESWA.2010.10.081.

[39]	R. N. Sharpe, M. yuen Chow, S. Briggs, and L. Windingland, “A Methodology Using Fuzzy Logic to Optimize Feedforward Artificial Neural Network Configurations,” IEEE Transactions on Systems, Man and Cybernetics, vol. 24, no. 5, pp. 760–768, 1994, doi: 10.1109/21.293489.

[40]	A. Ergin, E. Karaesmen, A. Micallef, and A. T. Williams, “A new methodology for evaluating coastal scenery: fuzzy logic systems,” Area, vol. 36, no. 4, pp. 367–386, Dec. 2004, doi: 10.1111/J.0004-0894.2004.00238.X.

[41]	M. Sigala, A. Beer, L. Hodgson, and A. O’Connor, Big Data for Measuring the Impact of Tourism Economic Development Programmes: A Process and Quality Criteria Framework for Using Big Data. 2019.

[42]	G. Nguyen et al., “Machine Learning and Deep Learning frameworks and libraries for large-scale data mining: a survey,” Artificial Intelligence Review, vol. 52, no. 1, pp. 77–124, 2019, doi: 10.1007/s10462-018-09679-z.

[43]	C. Shorten and T. M. Khoshgoftaar, “A survey on Image Data Augmentation for Deep Learning,” Journal of Big Data, vol. 6, no. 1, 2019, doi: 10.1186/s40537-019-0197-0.

[44]	R. Vinayakumar, M. Alazab, K. P. Soman, P. Poornachandran, A. Al-Nemrat, and S. Venkatraman, “Deep Learning Approach for Intelligent Intrusion Detection System,” IEEE Access, vol. 7, pp. 41525–41550, 2019, doi: 10.1109/ACCESS.2019.2895334.

[45]	K. Sivaraman, R. M. V. Krishnan, B. Sundarraj, and S. Sri Gowthem, “Network failure detection and diagnosis by analyzing syslog and SNS data: Applying big data analysis to network operations,” International Journal of Innovative Technology and Exploring Engineering, vol. 8, no. 9 Special Issue 3, pp. 883–887, 2019, doi: 10.35940/ijitee.I3187.0789S319.

[46]	A. D. Dwivedi, G. Srivastava, S. Dhar, and R. Singh, “A decentralized privacy-preserving healthcare blockchain for IoT,” Sensors (Switzerland), vol. 19, no. 2, pp. 1–17, 2019, doi: 10.3390/s19020326.

[47]	F. Al-Turjman, H. Zahmatkesh, and L. Mostarda, “Quantifying uncertainty in internet of medical things and big-data services using intelligence and deep learning,” IEEE Access, vol. 7, pp. 115749–115759, 2019, doi: 10.1109/ACCESS.2019.2931637.

[48]	S. Kumar and M. Singh, “Big data analytics for healthcare industry: Impact, applications, and tools,” Big Data Mining and Analytics, vol. 2, no. 1, pp. 48–57, 2019, doi: 10.26599/BDMA.2018.9020031.

[49]	L. M. Ang, K. P. Seng, G. K. Ijemaru, and A. M. Zungeru, “Deployment of IoV for Smart Cities: Applications, Architecture, and Challenges,” IEEE Access, vol. 7, pp. 6473–6492, 2019, doi: 10.1109/ACCESS.2018.2887076.

[50]	B. P. L. Lau et al., “A survey of data fusion in smart city applications,” Information Fusion, vol. 52, no. January, pp. 357–374, 2019, doi: 10.1016/j.inffus.2019.05.004.

[51]	Y. Wu et al., “Large scale incremental learning,” Proceedings of the IEEE Computer Society Conference on Computer Vision and Pattern Recognition, vol. 2019-June, pp. 374–382, 2019, doi: 10.1109/CVPR.2019.00046.

[52]	A. Mosavi, S. Shamshirband, E. Salwana, K. wing Chau, and J. H. M. Tah, “Prediction of multi-inputs bubble column reactor using a novel hybrid model of computational fluid dynamics and machine learning,” Engineering Applications of Computational Fluid Mechanics, vol. 13, no. 1, pp. 482–492, 2019, doi: 10.1080/19942060.2019.1613448.

[53]	V. Palanisamy and R. Thirunavukarasu, “Implications of big data analytics in developing healthcare frameworks – A review,” Journal of King Saud University - Computer and Information Sciences, vol. 31, no. 4, pp. 415–425, 2019, doi: 10.1016/j.jksuci.2017.12.007.

[54]	J. Sadowski, “When data is capital: Datafication, accumulation, and extraction,” Big Data and Society, vol. 6, no. 1, pp. 1–12, 2019, doi: 10.1177/2053951718820549.

[55]	J. R. Saura, B. R. Herraez, and A. Reyes-Menendez, “Comparing a traditional approach for financial brand communication analysis with a big data analytics technique,” IEEE Access, vol. 7, pp. 37100–37108, 2019, doi: 10.1109/ACCESS.2019.2905301.

[56]	D. Nallaperuma et al., “Online Incremental Machine Learning Platform for Big Data-Driven Smart Traffic Management,” IEEE Transactions on Intelligent Transportation Systems, vol. 20, no. 12, pp. 4679–4690, 2019, doi: 10.1109/TITS.2019.2924883.

[57]	S. Schulz, M. Becker, M. R. Groseclose, S. Schadt, and C. Hopf, “Advanced MALDI mass spectrometry imaging in pharmaceutical research and drug development,” Current Opinion in Biotechnology, vol. 55, pp. 51–59, 2019, doi: 10.1016/j.copbio.2018.08.003.

[58]	C. Shang and F. You, “Data Analytics and Machine Learning for Smart Process Manufacturing: Recent Advances and Perspectives in the Big Data Era,” Engineering, vol. 5, no. 6, pp. 1010–1016, 2019, doi: 10.1016/j.eng.2019.01.019.

[59]	Y. Yu, M. Li, L. Liu, Y. Li, and J. Wang, “Clinical big data and deep learning: Applications, challenges, and future outlooks,” Big Data Mining and Analytics, vol. 2, no. 4, pp. 288–305, 2019, doi: 10.26599/BDMA.2019.9020007.

[60]	M. Huang, W. Liu, T. Wang, H. Song, X. Li, and A. Liu, “A queuing delay utilization scheme for on-path service aggregation in services-oriented computing networks,” IEEE Access, vol. 7, pp. 23816–23833, 2019, doi: 10.1109/ACCESS.2019.2899402.

[61]	G. Xu, Y. Shi, X. Sun, and W. Shen, “Internet of things in marine environment monitoring: A review,” Sensors (Switzerland), vol. 19, no. 7, pp. 1–21, 2019, doi: 10.3390/s19071711.

[62]	M. Aqib, R. Mehmood, A. Alzahrani, I. Katib, A. Albeshri, and S. M. Altowaijri, Smarter traffic prediction using big data, in-memory computing, deep learning and gpus, vol. 19, no. 9. 2019.

[63]	S. Leonelli and N. Tempini, Data Journeys in the Sciences. 2020.

[64]	N. Stylos and J. Zwiegelaar, Big Data as a Game Changer: How Does It Shape Business Intelligence Within a Tourism and Hospitality Industry Context? 2019.

[65]	Q. Song, H. Ge, J. Caverlee, and X. Hu, “Tensor completion algorithms in big data analytics,” arXiv, vol. 13, no. 1, 2017.

BIOGRAPHIES OF AUTHORS

|  | D. Desmira is PhD student in Faculty of Electrical Engineering, Universiti Teknikal Melaka, Malaysia. She received his master degree in Electrical Engineering from Universitas Gunadarma Indonesia and his bachelor degree in electrical engineering from Universitas Muhammadiyah Yogyakarta, Indonesia. In 2014 she joined the Department of Electrical Engineering Vocational Education in Faculty of Teacher Training and Education of Universitas Sultan Ageng Tirtayasa, Serang Indonesia as lecturer. He has written several papers in the areas of electrical engineering, sensors, electronics, etc. |
| --- | --- |
|  | Norazhar Abu Bakar received his BEng. (Hons.) degree in electronics and electrical engineering from Leeds University, UK, MSc. (Eng.) in control systems from Sheffield University, UK, and PhD degree in electrical and computer engineering from Curtin University, Australia. He started his career as an assembly engineer at Toshiba Electronics, Malaysia, and then moved to TNB as a SCADA/DA Project Engineer. Currently, he is the Deputy Dean (Research & Postgraduate Studies) with the Faculty of Electrical Engineering, UTeM. He is a member of the High Voltage Engineering Research Laboratory in UTeM. His research interests are condition monitoring, asset management, and artificial intelligence. Since 2013, he has published several articles related to transformer condition monitoring and asset management in IEEE Transactions on Dielectrics and Electrical Insulation and IEEE Electrical Insulation Magazine. |
|  | Mohd Ruzaini Hashim is lecturer in Universiti Teknikal Malaysia Melaka (UTeM). |
|  | Romi Wiryadinata is lecturer in Department of Electrical Engineering, Universitas Sultan Ageng Tirtayasa. |
|  | Mustofa Abi Hamid received his master degree in Technical and Vocational Education from Universitas Negeri Padang (UNP) Padang Indonesia and his bachelor degree in physics education from Universitas Lampung (UNILA), Bandarlampung, Indonesia. In 2016 he joined the Department of Electrical Engineering Vocational Education in Faculty of Teacher Training and Education of Universitas Sultan Ageng Tirtayasa, Serang Indonesia as assistant professor. He has written several papers in the areas of electrical engineering, electronics, technical and vocational education, curriculum design, learning strategies, learning media, ICT for learning. |